D593/P/16/2/4/20  


An Estimate for the [ye] workmanship of the carcase of a Building design[e]d

for a Library and Servants Room, for the Right Hon[oura]ble Lord Govenor? at

Trentham in Staffordshire according to Drafts given in J? Francis Smith

Viz: The said Building to be of such length, and width, and walls of such thick

ness as figur[e]d on the [ye] plan and the [ye] rooms of such hight as figured on the section, one side and one end up to the water table to be built w[i]th

stone, and to worke and sett on venetian window of stone according to

the Draft and molds made for that purpose, to have a cornice of stone

along the [ye] two ends, and one side of the building, and to return one foot at

each end; The door next the house to have an architrave, fryze cornice

and pedyment of stone. To have a molded ground table along three

sides of the building and stone quines up the [ye] corners of the [ye] building,

to have two trusses und[e]r the Venetian Window w[i]th a cill and panell[e]d

pillasters to that window, and a beed up the [ye] inside of the jambs in stone,

the peers in the venetian window to be all stone; To have a stone plinth

on the [ye] cornice fourteen inches thick to run along along the backside, to have a

base mold run along the [ye] three sides and to return two foot on the [ye] backside.

To have stone pedestalls , raile, ballisters along the [ye] two ends, and

one side and a stone copeing along the [ye] backside to answer the [ye] raile

to have four window cills of stone and all the said stone worke to be

handsomely wrought according to the Draft and Molds;  .................

All the rest of the walls to be built w[i]th brick, and to make brick peers, and

contors, andturn arches over the [ye] servants room as  the [ye] draft directs, to frame

and put up a single joyst floor over the [ye] said Arch, and a cielding floor over the [ye]

Library, and a good substantiall roof over the [ye] said building and to board

the [ye] same fitt for leading; to make four transum windows for the [ye] said

Building, and to make a strong six pannell[e]d oake door and case into the

[page 2]
Library, and another into the Servants Room, and three sashes and

frames for the [ye] Venetian Windows; ............................

To have brick, stone, lime, sand, timber ready [saw[e]d ?] boards, nailes

and all scaffolding trickleropes and ladders and all oth[e]r materialls

found, and deliver[e]d convenient to the building my Lord to pay for

diging and throwing out  the earth, and finding the [ye] lead and for casting

and laying the [ye]  same; then ye? to performe all the affores[ai]d workmansh[i]p

after the best maner will be worth One Hundred and Three Pounds

Ten Shillings ................................... £103 – 10 – 0

75 Hundredweight or thereabout of lead will be wanting to

cover the [ye] sroof, lead and work if at 17/6 p[e]r hun[re]d is 65 – 12 – 6

London Crown glass for the [ye]  Benetian Window, and comon glass and

Casements for the [ye] oth[e]r windows will come to ..............6 – 0 – 0

